| CODEDU’s Students’ Training Course |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Section 3: Integrating Arduino with Academic Subjects |  |  |  |  |  |  |
| Subject: Arduino in STEM | Duration (in hours): 15 lesson hours |  |  |  |  |  |
| Target audience: Upper-primary and Secondary School Students |  |  |  |  |  |  |
| Training methodology: Online and Flipped Learning, Demonstration |  |  |  |  |  |  |
| Level (and cycle, if applicable) of the learning experience: Upper-primary and Secondary School Level (EQF3) |  |  |  |  |  |  |
| Assessment method: Multiple Questions |  | Form of participation in the learning activity: |  |  |  |  |
| Expected Learning outcomes: •Mastering basic electronics information • Defining basic Arduino sensors • Designing simple circuits with Arduino • Acquiring programming skills with Arduino • Acquiring project development skills in other disciplines with Arduino |  | - PowerPoint presentation |  |  |  |  |
| Prerequisites needed to enrol in the learning activities (if needed): |  |  |  |  |  |  |
| Supervision and identity verification during an assessment: |  |  |  |  |  |  |
| • Unsupervised with no identity verification. |  |  |  |  |  |  |
| • Supervised with no identity verification. |  |  |  |  |  |  |
| • Supervised online or onsite with identity verification. |  |  | X |  |  |  |
| Further information: Key Words Arduino, LDR, LED, servo motor, DHT11, LCD, electric, ohm law, voltage, conductor and insulator, sensor, ampere, ultrasonic sensor. |  |  |  |  |  |  |

Module 3.1

Arduino in STEM Education

Introduction: In the last decade, STEM education has become increasingly popular and is considered one of the most important emphases in contemporary science education reform movements. STEM (science, technology, engineering and mathematics) education involves the integration of various disciplines, primarily four disciplines. It is argued that STEM education is vital for students to develop 21st century skills such as problem solving, innovation, creativity, communication and collaboration. For this reason, STEM education has become important in bringing students to a level where they can compete globally in the 21st century. In STEM education, students seek solutions to real-life problems. Problems encountered in real life have an interdisciplinary structure and cannot be limited to the knowledge and skills of a particular discipline. Therefore, students should use the interdisciplinary approach of the knowledge and skills of different disciplines to solve these problems. Many researchers emphasize that an integrated education where interdisciplinary boundaries are removed is needed for an effective STEM education. At the core of STEM education is an inquiry approach that directs students to research, produce and make new discoveries. Therefore, STEM learning environments should be organized in this direction and students should be provided with opportunities to develop 21st century skills such as problem solving, critical thinking and creativity. In this context, STEM education is a process in which students collaborate with scientific inquiry and engineering design in collaborative environments to solve real-life problems. In this process, teachers direct students to question the main problem they encounter and provide students with the opportunity to find solutions to the problem on their own. 3.1.1. Applying Arduino to learn science, technology, engineering, and math concepts. What is Electricity? Electricity is the general name given to physical events related to the movement of charged particles. Electricity, as we know it, can be related to the current conduction of conductive materials, as well as different physical phenomena (events) such as static electricity, radio waves (electromagnetic waves) and lightning are also related to electricity. Electricity is basically formed by the movement of electrical current. Electric current is a current formed by free electrons moving along a conductor. This current operates devices and provides various applications by carrying electrical energy. Conductor and Insulator Materials Electricity is the movement of charged particles. This movement can occur on conductive materials. Materials that conduct electric current are called conductors. Metals are the most ideal conductors. On the other hand, materials that do not have free electrons to carry electric current are called insulator materials. In other words, materials that conduct electricity are called conductors, and materials that do not conduct electricity are called insulators. What is Electric Voltage? Electrical voltage is the electric field force required for the formation of current, that is, the movement of electrons. It allows the electric current to "flow", that is, to form. Its unit is Volt. It is shown by the letter V. Electrical voltage is the potential difference between two points in an electrical circuit, which allows current to flow through the circuit. Electrical voltage is usually measured in volts and determines how current flows through conductors. Voltage is a fundamental force that allows current to flow in a circuit. A voltage difference causes free electrons to move from one point to another, creating a current. In other words, there is no current without voltage. Voltage acts as a force that directs the current and determines its magnitude. In an electrical circuit, the voltage source is usually a device such as a battery or a generator. This voltage source creates a potential difference in the circuit, and this difference causes the current to move in a particular direction. The higher the voltage, the greater the current can be on the same conductor. The magnitude of the current is directly related to the voltage. According to Ohm's Law, the current in a circuit depends on the voltage and the resistance of the circuit. As the voltage increases, the current increases under the same resistance. This is a fundamental relationship for understanding how electrical energy is distributed and used. Voltage is measured with a voltmeter. The voltmeter is connected in parallel to the circuit element to be measured. When choosing a power source, we need to use a source that is suitable for the voltage at which our circuit will operate. For example, let's say we have a robot with 3V motors. To operate this robot, we will need an adapter or battery (for example, 2 AA batteries) that can provide 3V voltage. What is Electric Current? Electric current can be defined as the movement of electric charges through a conductive material as a result of the effect of potential difference (voltage). The moving charges are electrons in the material. Its unit is Ampere (A). Current is what does the real work in electrical circuits. Voltage ensures that the current flows. When choosing the battery or power source that will feed our circuit, we need to consider the highest current that the circuit will draw. If we think again about the robot example we gave when talking about voltage, let's assume that our robot operating at 6V will draw 1A current at its highest. In this case, we will need to use an adapter or battery that can provide at least 6V 1.5A current. The high current that our adapter can provide is not a problem for us, our robot will draw the 1A current it needs, and the adapter will easily meet this value. In other words, if we want, we can also use a 6V 10A power supply, only the cost and size of the power supply will increase. Current is measured with an ammeter. The ammeter is connected to the circuit in series. Electrical Circuit Electrical circuit; It is the name given to the system formed by connecting circuit elements such as resistors, capacitors, transistors, switches, generators, etc. together. Ohm's Law and Resistance We use conductive cables to transfer electric current. Again, if we go with the water example, we can imagine the cables as pipes carrying water. We can guess that as the diameter of these pipes narrows, the water that will flow will have more difficulty. In the electrical sense, resistance shows the same effect. The effect that occurs against electric current is called resistance. Its unit is Ohm. Its symbol is Ω (capital omega). It is expressed with the letter R in equations. Ohm's law states that the voltage (V) of a circuit element will be directly proportional to the current (I) passing through it and its resistance (R): V = I x R Sensors Various automations and smart projects are realized by combining sensors or sensors. Industry 4.0, a popular topic today, has emerged with advanced sensor technologies and continues to develop. Therefore, sensor technologies are a concept that pioneers the development of most projects and is gaining more importance every day. A sensor is a device that detects a physical or environmental change and converts this change into an electrical signal. Sensors are used in various applications and play a critical role in the modern technology world. A sensor is usually used to monitor, measure the environment around a device and create a response with this information. Sensors come in many different types, and each sensor is designed to detect a specific type of change. For example, a temperature sensor detects temperature changes in the environment and produces an electrical signal accordingly. Similarly, a pressure sensor detects the pressure applied to it and converts it into a signal. A sensor is a device that detects an environmental or physical change and converts this change into an electrical signal. The working principle of sensors is based on expressing the changes they detect with electronic signals. These signals are processed by the device or system to which the sensor is connected, and a specific response is generated. The working principle of a sensor varies depending on its type and the physical quantity it detects. For example, a temperature sensor detects the temperature in its environment and expresses this temperature change as a change in resistance, voltage or current. This temperature sensor transmits the changes it detects to a microcontroller or other electronic system, allowing the processing of temperature-related data. As a simple example, let's consider light sensors. A light sensor detects the light intensity in the environment and converts this information into an electrical signal. The light sensor usually works using a photodiode or photoresistor. When the amount of light in the environment increases, the resistance of the sensor decreases, and this change causes a voltage change in the electronic circuit to which it is connected. This voltage change allows the sensor to produce a signal depending on the light intensity. The most used sensors in robotic systems are: • Distance sensors (Ultrasonic, PIR) • Force/Weight/Pressure sensors • Tilt sensors • Magnetic sensors • Temperature/Humidity/Water Level sensors • Sound sensors • Light/Color sensors

| Activity |  |
| --- | --- |
| Activity 1 | Hands On Application-1 Project Name: Self-Extinguishing Street Lamp In The Dark |
|  | Learning outcomes: After this module you will be able to: - Establishing simple circuit skills, - Understanding energy saving, environmental awareness and sustainability, - Understanding electrical circuits, which is one of the physics subjects. Description: In this project, we measure the light level using the light-sensitive LDR sensor and apply an LED that automatically lights up in a dark environment. Level : Upper-primary and Secondary School Students Duration : 40 min + 40 min Learning Subject: - Application Development with Arduino - The sensors used in the robotic system, which produces a solution to a problem situation, make a difference. - It gives examples from daily life of the use of different sensors. - Providing the control system of the robot and designing intelligent systems - It uses sensors that work. Basic Skills: - Problem solving, - Analytical thinking, - Discover - Interdisciplinary thinking. Preparation & Research: It is very important that the streets are illuminated at night. But do these lamps need to be on all the time? How can we make the operation of these lamps more economical? How do you think we can use robotic technologies for this? Knowledge & Science: The headlights of some cars turn on automatically when entering a tunnel or when it gets dark. If it is daytime or light, the headlights are off. The automatic flashing of the headlights is achieved by measuring the light level. Photosensitive sensors are used to measure the intensity of light. LDR (light dependent resistor) means light-dependent resistor and is an analog circuit element. Therefore, Arduino is connected to the analog pins of the board. LDR is made of semiconductor materials that could absorb photons of light. As the light intensity falling on the LDR decreases, the resistance value increases, and as the light intensity increases, the resistance value decreases. You can use LDR together with the LDR module in our circuit. The LDR module has the module's power pins and data pin. In this coding project, we measure the light intensity using LDR and design a lighting system that lights up when the light intensity falls below the reference value we have set. What do you need: - Arduino Uno - LDR sensor - LED - 1K Ohm resistor - Breadboard - Jumper cable Design: In this part of the event there is the creation of a circuit containing Arduino. First, the electronic components that make up the circuit are located in the Tinkercad environment, added to the circuit and the necessary wiring made. Conclusion: In this coding project, we measured the light intensity of the environment using the Arduino UNO board and LDR sensor. We displayed the measured values on the serial port screen and determined a reference light intensity value for the dark environment by measuring the level of light intensity when the environment was dark. Finally, according to this reference value, we flashed the LED. With this application, you can develop different projects that will save energy, such as street lights that automatically turn on when it gets dark. |
|  | Sources: - https://maker.robotistan.com/gerilim-akim-direnc-nedir-temel-elektrik-kavramlari/, Access date: 28.12.2024 |
|  | Extra contents: |

| Activity |  |
| --- | --- |
| Activity 2 | Hands On Application-2 Project Name: Object Detection with Ultrasonic Distance Sensor |
|  | Learning outcomes: After this module you will be able to: - Establishing simple circuit building skills, - Acquiring skills in measuring distances and operations, which is one of the subjects of mathematics. Description: We make an object-sensing circuit using an Arduino UNO board and an ultrasonic sensor. Level : Upper-Upper-primary and Secondary School Students Duration : 40 min + 40 min Learning Subject: - Application Development with Arduino - The sensors used in the robotic system, which produces a solution to a problem situation, make a difference. - It gives examples from daily life of the use of different sensors. - Providing the control system of the robot and designing intelligent systems - It uses sensors that work. Basic Skills: - Problem solving, - Analytical thinking, - Discover - Interdisciplinary thinking. Preparation & Research: Today, in shopping malls, hospitals, etc., the doors open automatically when adults approach, while the automatic door does not open when a child or cat passes. What kind of sensor do you think could be used?" Knowledge & Science: Bats use echo localization to be able to orient themselves and detect obstacles around them. Thanks to this, they can fly even in a completely dark cave without hitting obstacles. Sound waves from bats are reflected when they hit an obstacle or object. When bats hear the reflected sound, they can determine the location, size, and shape of the obstacle or object. In this coding project, we measure the distance using the Ultrasenic distance sensor and design the system that signals if the distance is above the reference value we have determined (the automatic door can also be opened, but only sound and light will be produced in our example). What do you need?: • Arduino Uno • HC-SR04 Ultrasonic sensor • Buzzer • RGB Led • 3 x 1K Ohm resistors • Breadboard • Jumper cable (Jumper cable) Design: The installation of the circuit on the board and the block-based representation of the codes are available below. Carefully install and code your circuit. Conclusion: When the simulation is run, the Distance sensor is clicked. The green round object on the sensor shown in Image 3 is moved left, right, in and out of the area where the distance sensor detects the object. The green round object is within the range of the motion sensor's view, while the distance of the green round object is more than 120 cm away. The sound from the buzzer stops and the RGB Led lights up Green. If the distance of the object is closer than 120cm, the RGB LED lights up red and a warning sound is observed from the Buzzer. |
|  | Sources: |
|  | Extra contents: |

Activity 3 (Assessment of Module 3.1)

Type: Multiple Question (5 point each question)

1. When an ultrasonic sensor measures the distance of an object, what happens when sound waves hit the object? - The sound waves are lost and the sensor does not give accurate results. - The sound waves do not reach the object directly, they only spread through the air. - The sound waves are reflected back and the sensor uses these reflected sound waves to calculate the distance. - The sound waves pass through the object and the sensor cannot measure the distance. 2. In a project with an LDR sensor, what happens as the light level increases? - The resistance of the LDR increases. - The resistance of the LDR decreases. - The LED light turns on. - The power of the Arduino decreases. 3. Which sensor is used to measure the light level of an environment? - LDR (Light Dependent Resistor) sensor - Ultrasonic sensor - Temperature sensor - Humidity sensor 4. According to Ohm's Law, what is the relationship between voltage (V) and current (I) in a circuit? - Voltage and current are inversely proportional to each other. - Voltage is directly proportional to current and resistance. - Voltage and resistance are independent of each other. - Voltage is inversely proportional to current and resistance.

Total Score: /10 Points Grading Scale: 15-20 points: Excellent understanding of project presentation and documentation. 10-14 points: Good understanding with minor gaps. 6-9 points: Basic understanding, but needs improvement. Below 5 points: Needs further review of module 3.1. Answers: C, B, A, B

Module 3.2

Cross-Disciplinary Applications

Introduction: Arduino is a user-friendly platform that enables students to design and build their own electronic projects. With a simple programming language and a wide range of hardware support, students can easily create complex electronic circuits. One of the most important features of Arduino is that it allows students to bring together different disciplines. In this way, students can utilize engineering design principles and perform mathematical calculations while using scientific methods to solve a problem. Cross-disciplinary practices in STEM education help students develop a more holistic approach to real-world problems. For example, designing an automatic irrigation system uses knowledge from different disciplines such as plant physiology (biology), physical properties of water (physics), sensor technology (electronics) and programming (computer science). In this way, you learn to see knowledge as a whole rather than in parts. 3.2.1 Using Arduino in art, music, and other subjects In recent years, Arduino has emerged as a powerful and versatile tool that bridges the gap between technology and creativity. Originally developed as an open-source electronics platform for prototyping, Arduino has transcended its technical roots and is finding applications in a variety of fields, including art, music, and education. By integrating sensors, actuators, and programming, Arduino allows students and creators to explore innovative solutions and push the boundaries of traditional disciplines. Arduino in Art Art is no longer limited to static canvases or traditional media. With Arduino, artists can add interactivity and dynamic elements to their creations. For example, proximity sensors can enable installations to respond to viewer movements, while Arduino-controlled LEDs can create captivating light shows that change colors or patterns. Projects such as kinetic sculptures that incorporate motors and servos provide fluid movement in artworks. What’s more, Arduino’s compatibility with a variety of input devices, such as microphones and touch sensors, allows artists to create responsive artworks that interact with sound, touch, or environmental conditions. One notable application is interactive installations in public spaces. For example, an Arduino-powered mural can illuminate specific patterns based on sound levels or the presence of passersby, creating a dynamic connection between the artwork and its audience. This integration encourages interaction and elevates the traditional viewing experience to a participatory one. Arduino in Music The use of Arduino in music has revolutionized the way we create and experience sound. Arduino-based musical instruments have made it possible for anyone to design and build their own unique instruments. Using sensors such as piezoelectric elements, potentiometers, or capacitive touch inputs, creators can generate sounds or control pitch, volume, and effects. For example, Arduino can be used to create theremins, MIDI controllers, or drum pads. It can also enhance visual and auditory experiences by synchronizing light and sound in performances. One exciting project involves using a distance sensor to control the pitch of a note, allowing the performer to create melodies by simply moving their hand in the air. This combination of physical interaction and music production encourages creativity and inspires experimentation. 3.2.2. Collaborative and interdisciplinary projects One of the most powerful aspects of using Arduino in STEM education is its ability to foster collaborative and interdisciplinary projects. By working together on projects that integrate various disciplines, students gain valuable teamwork and communication skills while developing a deeper understanding of STEM concepts. Some examples of how collaborative and interdisciplinary projects can be implemented: - Automatic Opening Door in Case of Fire with Arduino: Students can develop a fire alarm system using fire alarm sensors, servo motors, and Arduino. This project combines different disciplines such as physics (motion, force), electronics (circuit design), computer science (programming), and even a little math (distance calculations). Students perform the necessary calculations and program the system so that the door opens at a certain angle when the fire alarm is triggered. - Seeing the Temperature and Humidity of the Classroom with Arduino: Students can connect temperature and humidity sensors to an Arduino to develop a system that continuously measures classroom air conditions. This project combines different disciplines such as physics (heat, humidity), electronics (sensors), computer science (data collection and analysis), and even environmental science (air quality). Students can analyse the data collected and make recommendations to improve classroom air quality. - LIGHT Control with LDR using Arduino: In this project, we will measure and control the light level required for healthy growth of plants in a greenhouse environment using Arduino. Using the light values measured by the light sensor, we will provide the optimum light level to the plants through LED lights. In this way, we can ensure that plants grow healthier and faster, and raise awareness about plant cultivation. - Rain sensitive smart clothesline system: In this project, we will design a system that detects rainfall using a water-resistant sensor and rotates the rope mechanism where the clothes are hung with the help of a motor to protect the clothes from rain.

| Activity |  |
| --- | --- |
| Activity 4 | Project Name: Automatic Opening Door in Case of Fire with Arduino |
|  | Learning outcomes: After this module you will be able to: - Understanding how to use a gas sensor, - Establishing simple circuit building skills, - Understanding electrical circuits, which is one of the physics subjects, - Acquiring distance and process measuring skills, which is one of the mathematics subjects. Description: Using an Arduino board, we design a door system that opens automatically when smoke or toxic gas is detected. Level : Upper-primary and Secondary School Students Duration : 40 min + 40 min Learning Subject: Application Development with Arduino - The sensors used in the robotic system, which produces a solution to a problem situation, make a difference. - It gives examples from daily life of the use of different sensors. - It uses sensors to design STEM applications and intelligent systems. Basic Skills: - Problem solving, - Analytical thinking, - Discover - Interdisciplinary thinking. Preparation & Research: Sometimes, when we leave the doors open, we find it difficult to maintain the temperature of the indoor space. For example, in winter, a heated place will cool down because of the door being open all the time and we will burn unnecessary fuel, or a door that is left open all the time in a place cooled by air conditioning will cause the environment inside to heat up. From here, we can understand that the doors need to be closed or opened depending on the situation. So why don't the doors open on their own in the event of a fire and save us time? Our daily life has now become even easier thanks to many systems that work automatically. Do you think motion-sensitive lights and doors that open according to the situation are beneficial? Knowledge & Science: During a fire, different gases are released when various types of substances in the environment catch fire. Some of these gases are toxic, some are suffocating, and some may be irritating. Gases are the greatest of the hazards in the fireplace. Gases generated because of fires, especially in closed areas, threaten human health. In fires that break out in closed areas, it may cause a stampede if the people inside head to the doors at the same time. If the doors are closed or opened inward, it can increase the stampede even more. For this reason, it is of great importance that the doors work automatically and open outwards in the event of a fire. What do you need: • Computer with Internet connection • Arduino UNO board • MQ gas sensor • Servo motor • Buzzer (Sound Generator) • Breadboard • Jumper cable Conclusion : In this event, we tested whether the system we designed on Tinkercad works using a real Arduino board. Using the gas sensor, we measured the amount of toxic gas in the environment. When the measured value was more than the specified threshold value, the door connected to the servo motor was opened. In this way, we representedly ensured both the ventilation of the indoor environment and the quick exit of the people inside. In addition, by making a noise with the buzzer, we announced to the people inside that there was poisonous gas and that they should evacuate the building as soon as possible. |
|  | Sources: |
|  | Extra contents: |

| Activity |  |
| --- | --- |
| Activity 5 | Seeing the Temperature and Humidity of the Classroom with Arduino |
|  | Learning outcomes: After this module you will be able to: - Gaining Sensor Usage and Data Reading Skills, - Gaining Data Visualization and LCD Screen Usage Skills, - Gaining Analytical Thinking and Problem Solving Skills, - Gaining Awareness of Understanding Environmental Factors, - Increasing Awareness of the Application of STEM Approach Description: Using an Arduino board, we want to develop a system that displays the temperature and humidity in the classroom. Level: Secondary School Students Duration : 40 min + 40 min Learning Subject: Application Development with Arduino - The sensors used in the robotic system, which produces a solution to a problem situation, make a difference. - It gives examples from daily life of the use of different sensors. - It uses sensors and imagers to design STEM applications and intelligent systems. Basic Skills: - Problem solving, - Analytical thinking, - Discover - Interdisciplinary thinking. Preparation & Research: We will examine the air of our classroom like a laboratory and discover its hidden world by measuring values such as temperature and humidity. So why do we want to measure temperature and humidity? The mood of our classroom can affect how comfortable we feel while studying. Studying in an environment that is too hot or too cold can be challenging. Likewise, the humidity of the air is important for our health. How can we improve the temperature and humidity values in our classroom? For example, can we consider different methods such as opening windows and using air conditioning? Knowledge And Science: In this project, we will measure the weather around us using a DHT11 temperature and humidity sensor and display it on a 16×2 LCD screen with I2C support. In this way, you will be able to monitor temperature and humidity values in real time. The DHT11 sensor is known for its low cost and easy operation. With a temperature accuracy of ±2°C and a humidity accuracy of ±5%, it is ideal for basic measurement needs. The I2C LCD display, on the other hand, reduces cabling complexity in projects by providing data display using fewer pins. What do you need?: - 1x Arduino (Uno, Nano, etc.) - 1x Breadboard - 1x DHT11 Temperature and Humidity Sensor - 1x I2C 16×2 LCD Display - Jumper Cables Design: The installation of the circuit on the board and the block-based representation of the codes are available below. Carefully install and code your circuit. Directions: - DHT11 Tempreture and Humidity Sensor - VCC → to Arduino 5V pin - GND → Arduino to GND pin - DATA → to Arduino D2 pin - I2C 16×2 LCD: - VCC → to Arduino 5V pin - GND → Arduino to GND pin - SDA → to Arduino A4 pin - SCL → to Arduino A5 pin Codes: Codes; Explanation. #include <dht11.h> #include <Wire.h> #include <LiquidCrystal_I2C.h>; Library required to read DHT11 sensor data We define the Wire library for the communication of Arduino and I2C LCD Display We make the library identification of our I2C LCD Display. Codes; Explanation. int temperature_value = 2; dht11 DHT11_sensor; LiquidCrystal_I2C lcd(0x27, 16, 2);; We connect the DHT11 sensor to the D2 pin of the Arduino We create object to use the sensor. We define the I2C address and size of the LCD screen (address: 0x27).. Codes; Explanation. void setup() { lcd.init(); lcd.backlight(); }; We start the LCD display We turn on the backlight of the LCD screen.. Codes; Explanation. void loop() { int chk = DHT11_sensor.read(temperature_value);; We read the temperature and humidity data from the sensor.. Codes; Explanation. lcd.setCursor(0, 0); lcd.print("Temperature= "); lcd.print((double)DHT11_sensor.temperature, 2);; We move on to the first line of the LCD screen. We print the temperature value on the screen.. Codes; Explanation. lcd.setCursor(0, 1); lcd.print("Humidity Rate = "); lcd.print((double)DHT11_sensor.humidity, 2); delay(1000); }; We move on to the second line of the LCD screen. We print the humidity value on the screen. We update the data with 1 second waiting time.. Codes View: //----------------------------------------------------------// //------------------CODEDU Project------------------// //--Temperature and Humidity with DHT11--// #include <dht11.h> #include <Wire.h> #include <LiquidCrystal_I2C.h> int temperature_value = 2; dht11 DHT11_sensor; LiquidCrystal_I2C lcd(0x27, 16, 2); void setup() { LCD.init(); LCD.backlight(); } void loop() { int chk = DHT11_sensor.read(temperature_value); LCD.setCursor(0, 0); LCD.print("Temperature= "); LCD.print((double)DHT11_sensor.temperature, 2); LCD.setCursor(0, 1); LCD.print("Humidity Rate = "); LCD.print((double)DHT11_sensor.humidity, 2); delay(1000); } Conclusion: In this project, we aimed to determine the most suitable working conditions by measuring the temperature and humidity values ​​of our classroom using the Arduino platform. We visualized the data we obtained on a graph and analyzed in detail how the temperature of our classroom changed during the day. As a result of our analysis, we determined that the most suitable working temperature of our classroom was between 22-24 degrees. Thanks to this project, we improved our data collection, analysis and visualization skills. In the future, we want to develop this project further and measure different environmental factors or design an automation system using the data we obtained. |
|  | Sources: |
|  | Extra contents: |

| Activity |  |
| --- | --- |
| Activity 6 | LIGHT Control with LDR using Arduino |
|  | Learning outcomes: After this module you will be able to: - Gaining Sensor Usage and Data Reading Skills, - Gaining Analytical Thinking and Problem-Solving Skills, - Gaining Awareness of Understanding Environmental Factors, - Increasing Awareness of the Application of STEM Approach. Description: In this project, we will measure and control the light level required for healthy growth of plants in a greenhouse environment using Arduino. Using the light values ​​measured by the light sensor, we will provide the optimum light level to the plants through LED lights. In this way, we can ensure that plants grow healthier and faster, and raise awareness about plant cultivation. Level: Secondary School Students Duration: 40 min + 40 min Learning Subject: Application Development with Arduino - The sensors used in the robotic system, which produces a solution to a problem situation, make a difference. - It gives examples from daily life of the use of different sensors. - It uses sensors and imagers to design STEM applications and intelligent systems. Basic Skills: - Problem solving, - Analytical thinking, - Discover - Interdisciplinary thinking. Preparation & Research: Plants grow and develop by converting sunlight into energy through a process called photosynthesis. However, not all plants have the same light needs. So, what are the light needs of different types of plants and why do they differ? Research the light needs of plants. What are the light needs of different types of plants? Knowledge And Science: A photo resistor is an element with a resistance that changes depending on the intensity of light falling on it. Its resistance changes inversely proportional to the amount of light falling on it. It is commonly used in many electronic devices we use daily under the name “photocell”. In this application, our 5 LEDs will work as a light level indicator by lighting up in different numbers according to the amount of light detected by the LDR. What do you need?: - Arduino (you can use any model you want) - Breadboard - 1 x 5mm LDR (Light Dependent Resistor) - 5 x Red LED - 5 x 330 Ohm Resistor (Orange-Orange-Brown) - 1 x 10k Ohm Resistor (Brown-Black-Orange) - 15 x Jumper Cables Design: The installation of the circuit on the board and the block-based representation of the codes are available below. Carefully install and code your circuit. Codes: int LEDs[] = { 2, 3, 4, 5, 6 }; int number_of_LED = 5; int ldr_pin = A0; int value = 0; void setup() { for (int i = 0; i < number_of_LED; i++) { pinMode(LEDs[i], OUTPUT); } } void loop() { value = analogRead(ldr_pin); if (value > 0 && value <= 255) { digitalWrite(LEDs [0], HIGH); digitalWrite(LEDs [1], LOW); digitalWrite(LEDs [2], LOW); digitalWrite(LEDs [3], LOW); digitalWrite(LEDs [4], LOW); } Else if (value > 256 && value <= 500) { digitalWrite(LEDs[0], HIGH); digitalWrite(LEDs[1], HIGH); digitalWrite(LEDs[2], LOW); digitalWrite(LEDs[3], LOW); digitalWrite(LEDs[4], LOW); } Else if (value > 501 && value <= 700) { digitalWrite(LEDs[0], HIGH); digitalWrite(LEDs[1], HIGH); digitalWrite(LEDs[2], HIGH); digitalWrite(LEDs[3], LOW); digitalWrite(LEDs[4], LOW); } Else if (value > 701 && value <= 900) { digitalWrite(LEDs[0], HIGH); digitalWrite(LEDs[1], HIGH); digitalWrite(LEDs[2], HIGH); digitalWrite(LEDs[3], HIGH); digitalWrite(LEDs[4], LOW); } Else digitalWrite(LEDs[0], HIGH); digitalWrite(LEDs[1], HIGH); digitalWrite(LEDs[2], HIGH); digitalWrite(LEDs[3], HIGH); digitalWrite(LEDs[4], HIGH); delay(100); } Conclusion: In this project, we developed a simple automation system to meet the light needs of plants in a greenhouse environment using the Arduino platform. The light values ​​measured by the light sensor were processed by Arduino and the brightness of the LED lights were controlled. In this way, the optimum light conditions required for healthy plant growth were provided. Throughout the project, we developed our knowledge and skills in electronic circuit design, Arduino programming and plant physiology. In addition, a more sensitive and personalized control system can be created by analyzing plant growth data using artificial intelligence algorithms. In this way, more efficient and sustainable production methods can be developed in the agricultural sector. This project offers an important opportunity to develop scientific thinking and application skills, especially for students. It also draws attention to sustainable agriculture and environmental awareness. Such projects are of great importance for training future engineers and scientists. |
|  | Sources: |
|  | Extra contents: |

| Activity |  |
| --- | --- |
| Activity 7 | Rain sensitive smart clothesline system |
|  | Learning outcomes: After this module you will be able to: - Gaining Sensor Usage and Data Reading Skills, - Gaining Analytical Thinking and Problem-Solving Skills, - Gaining Awareness of Understanding Environmental Factors, - Increasing Awareness of the Application of STEM Approach Description: In this sproject, we will design a system that detects rainfall using a water-resistant sensor and rotates the rope mechanism where the clothes are hung with the help of a motor to protect the clothes from rain. Level: Secondary School Students Duration : 40 min + 40 min Learning Subject: Application Development with Arduino - The sensors used in the robotic system, which produces a solution to a problem situation, make a difference. - It gives examples from daily life of the use of different sensors. - It uses sensors and imagers to design STEM applications and intelligent systems. Basic Skills: - Problem solving, - Analytical thinking, - Discover - Interdisciplinary thinking. Preparation & Research: In order to better understand the project, research and try to answer the following questions. - Which type of motor would be most suitable for this project? Servo motor or stepper motor? - How can we adjust the sensitivity of the rain sensor and prevent false triggers? - How can this system be used more efficiently in real life? - Can a system that can respond to different conditions other than rain be designed? - Is it possible to integrate this system into smart home systems? Knowledge And Science: Have you ever thought of a way to prevent a water tank from overflowing or remaining empty? Do you know that we can do this with Arduino? Let's say we want to control the water level in a water tank. If the tank is too full, the sensor sends a signal and stops the water. If the tank is empty, the sensor can send another signal to refill the water. This sensor is a device that detects the presence or height of water. If the sensor touches the water, it sends a signal to the Arduino and we can perform an action using this signal, such as sounding an alarm or starting a pump. In this project, we will design an application that makes the motor turn when rain is detected and returns the hanging laundry to a safe area. The working principle of the sensor used is as follows. If rain is detected, the output will be "LOW". If rain is not detected, the output will be "HIGH". Design: The installation of the circuit on the board and the block-based representation of the codes are available below. Carefully install and code your circuit. Codes: Codes; Explanation. #include <Servo.h> #define Rain_Sensor_pin A0 #define Servo_Motor_pin 9; Pin A0 is connected to the rain sensor Pin 9 is connected to the servo motor. Codes; Explanation. Servo servo_motor; int angle = 0; int prev_rain; int rain;; To control the Servo Motor an object named servo_motor is created A variable named “angel” was defined and the initial value was assigned as “0”. A variable named “prev_rain”was defined. A variable named “rain” was defined. Codes; Explanation. void setup() { Serial.begin(9600); pinMode(Rain_Sensor_pin, INPUT); servo_motor.attach(Servo_Motor_pin); servo_motor.write(angle); rain = digitalRead(Rain_Sensor_pin); }; Serial connection setting Arduino pin set to INPUT mode Servo motor connected to pin 9 The starting angle of the servo motor is set to 0 degrees The value read from the rain sensor is assigned to the "rain" variable.. Codes; Explanation. void loop() { prev_rain = rain; rain = digitalRead(Rain_Sensor_pin); if (rain == LOW && prev_rain == HIGH) { Serial.println("Rain detected! The motor returns the clothesline to the safe area."); servo_motor.write(90); } if (rain == HIGH && prev_rain == LOW) { Serial.println("No Rain! The motor returns the clothesline to its original position."); servo_motor.write(0); } }; The latest rain status is recorded New rain status is assigned to the “rain” If there is no rain in the old state and there is rain in the new state, send a message to the output and set the engine angle to 90 degrees. If there is rain in the old state and there is no rain in the new state, send a message to the output and set the motor angle to 0 degrees.. Code View: #include <Servo.h> #define Rain_Sensor_pin A0 #define Servo_Motor_pin 9 Servo servo_motor; int angle = 0; int prev_rain; int rain; void setup() { Serial.begin(9600); pinMode(Rain_Sensor_pin, INPUT); servo_motor.attach(Servo_Motor_pin); servo_motor.write(angle); rain = digitalRead(Rain_Sensor_pin); } void loop() { prev_rain = rain; rain = digitalRead(Rain_Sensor_pin); if (rain == LOW && prev_rain == HIGH) { Serial.println("Rain detected! The motor returns the clothesline to the safe area."); servo_motor.write(90); } else if (rain == HIGH && prev_rain == LOW) { Serial.println("Rain stopped! The motor returns the clothesline to its original position."); servo_motor.write(0); } } Conclusion: In this project, we have successfully designed a rain detection system that automatically moves the clothesline to a safe position when rain is detected. The use of a rain sensor and a servo motor demonstrates how simple electronic components can solve real-life problems efficiently. This project also highlights the importance of integrating sensors and actuators into STEM applications, offering students the opportunity to enhance their problem-solving and analytical thinking skills. Furthermore, this system could be further developed to handle other environmental conditions or integrated into smart home systems, making it a versatile and practical solution for everyday life. |
|  | Sources: |
|  | Extra contents: |

Activity 8 (Assessment of Module 3.2)

Type: Multiple choice question (5 points per each question)

1. Which condition triggers the servo motor to move the clothesline to a safe area? - When the rain sensor output changes from HIGH to LOW - When the rain sensor output changes from LOW to HIGH - When the servo motor angle is set to 0 degrees - When the Serial Monitor is not connected 2. Why is a 10k Ohm resistor used in the circuit with the LDR? - To limit the current flowing through the LEDs - To stabilize the voltage divider circuit for the LDR - To protect the Arduino board from overvoltage - To increase the brightness of the LEDs 3. What is the role of the I2C interface in the LCD? - To measure the temperature values - To simplify wiring by using fewer pins for communication - To increase the accuracy of humidity measurements - To control the power supply of the Arduino

Total Score: /15 Points Grading Scale: 10-15 points: Excellent understanding of project presentation and documentation. 6-9 points: Good understanding with minor gaps. Below 5 points: Needs further review of module 3.2. Answers: A, B, B

| Assessment |  |
| --- | --- |
| Overall Assessment | 6/15 |